- 5.Sound test

- 1.Principle:

- Microphone is connected to the P1 of micro:bit and detects the current sound size by reading the analog value of the micro:bit P1. The larger sound of the current environment, the larger the analog value read, the smaller sound of the current environment, and the smaller the analog value read.

- Learning goals:

- In this lesson we will learn how to realize that when the larger sound of the current environment, the longer the graphic bar on the dot matrix; the smaller sound of the current environment, the fewer the graphic bars on the dot matrix.

- Programming method:

Online programming:

- First,we need to connect the micro:bit to the computer by USB data cable, the computer will pop up a USB flash drive.Then, click on the URL in the USB flash drive: http://microbit.org/ to enter the edit process interface.

Offilne programming:

- Open the offline programming software,download address of this software: http://www.microbitgo.com/code.

- After creating a new project, you need to load the Neopixel library to program the programmable RGB lights. To load the library, click on 【Advanced】—【Extensions】 — click on 【Neopixel】, and you will see an extra column in the programming interface. As shown in the figure below.

- General program diagram:

- Our programmable RGB lamp is connected to the pin2 of micro:bit, so the pin2 is set to no pull-up/pull-down mode by default. Then use the following statement to create a new NeoPixel driver named Strip.

- The initialize strip is set to black, meaning that the RGB light is turned off.

- Then in the main loop, we need to use the draw graph bar statement to display the analog value of the read P1 as a graph bar on the dot matrix. The maximum value is 1023.

- 4.Download program

- We need to make sure that the micro:bit board is connected to the computer. Click the download in the lower left corner as shown below, and select the download path as micro:bit drive letter to download the program.

- 5.Experimental phenomena

- After the program is successfully downloaded, you can see that fewer the graphic bars on the dot matrix. When we increase the sound or take a breath to microphone, the graphic bar will grow. As shown in the figure below.